Solar Energy Terminology and Assumptions

# 1. Why These Terms Matter

Before doing calculations or building the insurance trigger, the case study provides a few important solar-system terms and assumptions. These values help us understand what the system is expected to produce and how realistic electricity generation is estimated.

3 kVA

AEP50 = 4600 kWh/year

Performance Ratio (PR) = 80%

These values describe different things. They should not be confused with one another.

# 2. What Is 3 kVA?

Think of 3 kVA as the size or capacity rating of the rooftop solar system.

A simple comparison is:

| Device | Size / Rating |
| --- | --- |
| Bike | 150 cc |
| Air conditioner | 1.5 ton |
| Solar system | 3 kVA |

Just as 150 cc describes the size of a bike engine and 1.5 ton describes the rating of an air conditioner, 3 kVA describes the capacity rating of the installed solar system.

In this case study, 3 kVA tells us how large the solar system is. It is not the same thing as the amount of electricity produced over a year.

# 3. What Is 4600 kWh/year?

4600 kWh/year is the expected amount of electrical energy generated by the solar system in one year.

A simple water-tank analogy helps:

Tank size = capacity

Water supplied during a year = amount of water delivered

Similarly:

3 kVA = capacity or size of the solar system

4600 kWh/year = expected electricity generated during one year

One describes the size of the system. The other describes the energy it is expected to produce.

# 4. Difference Between kVA and kWh

| kVA | kWh |
| --- | --- |
| Capacity / size of the system | Electricity generated |
| System rating | Energy output |
| Represents installed capacity | Represents energy produced over time |

kVA tells us how big the solar system is, while kWh tells us how much electricity it produced.

# 5. What Is AEP50?

AEP stands for Annual Energy Production.

It represents the amount of electricity the solar system is expected to generate over one year.

The “P50” part means there is a 50% probability that the actual annual generation will be equal to or greater than this value.

For this case study:

AEP50 = 4600 kWh/year

This means that 4600 kWh/year is used as the expected annual electricity-generation benchmark for the 3 kVA solar system.

# 6. Why Is AEP50 Important?

AEP50 gives us a benchmark against which actual generation can be compared.

Example:

| Measure | Value |
| --- | --- |
| Expected annual generation | 4600 kWh/year |
| Actual generation | 3900 kWh/year |

Because we have the expected benchmark, we can see that actual generation was lower than expected.

Without AEP50, we would have much less context for deciding whether a particular year's generation was above or below the expected level.

# 7. What Is the Performance Ratio (PR)?

Solar systems do not convert all of the theoretical solar energy into usable electricity. Real-world losses reduce the output.

Some causes of loss include:

Heat

Wiring losses

Inverter losses

Dust

Other system inefficiencies

The Performance Ratio (PR) is used to account for these practical losses when estimating electricity generation.

For this case study:

PR = 80%

A simple way to think about it is that if the ideal conditions suggested 100 units of possible output, the estimate would use about 80 units after accounting for real-world losses.

# 8. Why Did the Case Study Provide PR = 80%?

The case study provides a Performance Ratio so the solar-generation estimate is more realistic than an ideal 100%-efficient system.

It represents the assumption that the system experiences normal real-world losses such as heat, wiring, inverter inefficiency, and dust.

# 9. How These Terms Work Together

Each value has a different role:

| Term | What It Tells Us |
| --- | --- |
| 3 kVA | The capacity rating of the installed solar system. |
| AEP50 = 4600 kWh/year | The expected annual electricity-generation benchmark. |
| PR = 80% | The assumption used to account for real-world system losses. |

## Q1. What does 3 kVA represent?

3 kVA represents the capacity rating of the installed rooftop solar system.

## Q2. What is 4600 kWh/year?

It is the expected amount of electricity generated by the solar system over one year.

## Q3. What is AEP50?

AEP stands for Annual Energy Production. AEP50 is the expected annual electricity-generation benchmark with a 50% probability that actual annual generation will be equal to or greater than that value.

## Q4. Why was AEP50 provided in the case study?

It provides the expected annual electricity generation, which acts as a benchmark for comparing actual performance.

## Q5. What is the Performance Ratio?

The Performance Ratio accounts for real-world system losses such as heat, wiring, inverter losses, and dust, so the estimated generation is more realistic.

## Q6. Why is PR assumed to be 80%?

Solar systems do not operate at 100% efficiency in real life. The 80% assumption accounts for practical losses and provides a more realistic estimate of electricity generation.

## Q7. What is the difference between kVA and kWh?

kVA describes the capacity rating of the solar system, while kWh describes the amount of electrical energy generated.